Quad Precision

Since the CPU is a 64-bit machine with 64-bit registers some means must be arrived at to perform 128-bit quad precision operations. The solution used is to perform the operation using register pairs. Even / Odd numbered register pairs are used.

Reservation station entries accommodate values up to 128-bits in size.

Argument valid bits are split into low and high bits.

For 128-bit arguments both bits must be valid.

For 64-bit argument only the low valid bit need be set.

Instructions are queued with the low order 64-bits of an argument, the upper 64-bits will be always be invalid.

The upper bits are supplied by request from the reservation station.

Interrupts

Even if software disables interrupts it may be ignored. The issue is due to pipelining. An interrupt causes immediate transfer of the fetch address to the ISR. So, interrupt instructions are in the pipeline as soon as possible. If there is a prior instruction still in the pipeline that disables interrupts, it is too late, the interrupt has already happened. The simplest solution is to ignore the disabling of interrupts. The interrupt mask is set in the fetch stage when an interrupt occurs. Only a higher priority interrupt is then able to interrupt. So, the interrupt must have had a higher priority than the routine currently running. Meaning lower-level interrupts were already disabled.

Predicate Mask

The predicate mask needs to be applied to all micro-ops of the same instruction. The mask cannot shift until the lead micro-op of the next instruction is seen. This makes the predicate masking logic non-trivial. The predicate bits have to be extracted based on the current instruction number of the predicate shadow.

The PRED instruction may occur more than once in the pipeline so there needs to be more than one predicate register. The core assigns a predicate register number in a circular fashion as PRED modifiers are encountered. The predicate register number to use is copied to all the following instructions.

Instruction Dispatch

There are now five dispatch slots which are loaded from the ROB and travel to the reservation stations. Previously there were only four slots. However, the previous slots were able to accept input from any ROB entry of any type of functional unit. This led to a lot of multiplexors begin used and the tools took a terribly long time to synthesize (they were stopped after two days). So, to reduce the multiplexer usage The slots now accept input for only specific functional units. This puts restrictions on which units can have  instructions dispatched in any given cycle. For instance, multiply and divide share a slot, so both a multiply and a divide cannot be dispatched in the same cycle. Previously they could be. To help compensate and extra slot was added.

Possible dispatches

1 SAU

1 Multiply or Divide

1 floating point op including trig and fused multiply-add

1 flow control op (branch)

1 memory op

Note that other than the SAU it is not as limiting as it seems, since many operations require multiple clock cycles.